# ENGR 103 Study Guide — Content Review Sheet

Golden Bear Engineering · Mid-Term edition · generated 2026-10-08

## How to use this sheet

- Start with the ⚠ items. These are the ones Claude is least sure of (course-specific wording, SolidWorks click paths, Pico/WNE103 details). Each is shaded gold.

- Then skim the rest. Only stop where an answer looks wrong or the wording does not match how you teach it.

- In the Check column write OK, or the fix (e.g. "answer is B", "delete", "reword: …"). A pen on a printout works fine.

- Send the ID numbers with the fixes (for example: "m05b → answer C; delete t10b") and Claude will correct everything in one pass.

- Questions marked [Figure: …] use drawings built into the page; view those on the live site, since the figure is not reproduced here.

- No answer keys were available, so every answer was worked out by Claude from the exams, slides and tutorials.

Items: 287 total · ⚠ flagged: 60

# Part A — ⚠ Check these first

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| ⚠ f04a | Card | The 5 steps of the design thinking process (in order) | 1) Empathize 2) Define 3) Ideate 4) Prototype 5) Test (Concepts are generated in Step 3, Ideate.) |  |
| ⚠ f04b | Card | Empathize | Understand the USER: observe, interview and immerse yourself in their situation. Goal: learn what they do, think, feel and struggle with, before proposing solutions. |  |
| ⚠ f04c | Card | Define | Turn what you learned into a clear problem (point-of-view) statement about the user's need, then list constraints and criteria for success. |  |
| ⚠ f04d | Card | Ideate | Generate MANY possible concepts (brainstorm, sketch), then screen and combine them to develop the best concept. Quantity first, judge later. |  |
| ⚠ f04e | Card | Prototype and Test | Prototype: build a quick, cheap model of the best concept (looks-like, works-like, or both). Test: try it with real users, learn what fails, and loop back to earlier steps to improve. |  |
| ⚠ f04f | Card | Is design thinking a straight line? | No. It is iterative. Testing results often send you back to Define or Ideate. Failing early and cheaply is part of the process. |  |
| ⚠ f05a | Card | How do you write a need (point-of-view) statement? | [A specific USER] needs [a way to do something / meet a need] because [surprising insight or reason]. Example: "A Friendly's server needs a way to portion ice cream fast and with one hand because the line backs up at rush hour." It states the need, NOT the solution. |  |
| ⚠ f05b | Card | What is a constraint? | A hard limit the design MUST satisfy (pass/fail): cost, size, time, materials, safety, standards. Example: "The scoop shall cost less than $25 per unit." |  |
| ⚠ f05c | Card | What is a criterion for success? | A measurable way to judge HOW WELL a design works, used to compare concepts. More is better (or less is better). Example: "Portions of ice cream differ by less than 10% in mass" or "cleans in under 30 seconds." |  |
| ⚠ f05e | Card | Good constraint statements are… | Written as a full sentence, specific, measurable/verifiable, and about the design (not a solution). e.g. "The scoop shall be cleanable without equipment other than water and soap." |  |
| ⚠ f06a | Card | Concept screening matrix: what is it for? | A quick way to narrow many concepts to a few promising ones by comparing each concept against a reference (datum) concept on each selection criterion. |  |
| ⚠ f06b | Card | Steps for using a screening matrix | 1) Prepare the matrix (criteria in rows, concepts in columns, pick a reference concept) 2) Rate each concept vs. the reference: + (better), 0 (same), − (worse) 3) Rank by net score 4) Combine and improve the concepts 5) Select / develop the best concept |  |
| ⚠ f06c | Card | Screening matrix vs. scoring matrix | Screening: coarse (+ / 0 / −) relative to a reference; used first to eliminate weak concepts. Scoring: finer (1–5 ratings) with WEIGHTS for each criterion; used on the survivors. Weighted score = Σ(weight × rating). |  |
| ⚠ f07a | Card | How are holes connected on a breadboard? | In the main area, the 5 holes in each short column strip (a–e or f–j) on the same numbered row are connected. The center gap separates the two halves. The long power rails run along the length of the board. |  |
| ⚠ f08b | Card | Servo wires | Signal: orange or yellow → a Pico GPIO pin (e.g. GP15) V+: red → external 4.8–6 V battery pack (+) GND: brown or black → battery pack (−) AND a Pico GND (common ground) |  |
| ⚠ f08e | Card | Wire a DC motor and battery to the motor shield | Motor leads → the motor terminals (Motor 1 / Motor 2) on the Kitronik shield. Battery pack → shield power terminals (+/− correct). Switch the shield ON. The Pico gets its own power (USB or the regulator to VSYS) and talks to the shield through the WNE103 module. |  |
| ⚠ f08f | Card | How does the shield control motor speed and direction? | Speed: Pulse Width Modulation (PWM). Low average current = slow, high = fast. Direction: an H-bridge circuit reverses the current through the motor (forward / reverse). |  |
| ⚠ f09c | Card | Read a digital input / analog value | Digital in: sw = Pin(14, Pin.IN, Pin.PULL_UP); sw.value() Analog in: from machine import ADC; a = ADC(26); a.read_u16() returns 0–65535. ADC pins are GP26–GP28. |  |
| ⚠ f09e | Card | Using the WNE103 motor module | import WNE103 board = WNE103.KitronikPicoMotor() board.motorOn(1, 'f', 100) # motor 1, forward, speed 100 board.motorOn(2, 'r', 100) # motor 2, reverse board.motorOff(1) |  |
| ⚠ f09f | Card | Make the robot turn | Pivot right: motor 1 forward, motor 2 reverse. Pivot left: motor 1 reverse, motor 2 forward. How far it turns depends on how long (time.sleep) you hold the turn. A 90° turn is found by testing. |  |
| ⚠ f09h | Card | What is main.py on the Pico? | The file the Pico runs automatically on power-up. Save your untethered robot code to the Pico as main.py. WNE103.py (library) is uploaded once. |  |
| ⚠ f10c | Card | Make a scatter plot in Excel | 1) Enter the X column (set speed) and Y column (velocity) 2) Select both columns → Insert → Scatter (markers only) 3) Add chart title, axis titles WITH UNITS 4) Optional: trendline |  |
| ⚠ f10e | Card | Memo format | Header block: TO, FROM, DATE, SUBJECT. Then a short body: purpose first, key details/results, and any needed action. Concise, single-spaced paragraphs, no letter-style greeting or closing. |  |
| ⚠ f11a | Card | Set document properties for a new part | Options → Document Properties: set Overall drafting standard to ANSI and Units to IPS (inch, pound, second) unless told otherwise. |  |
| ⚠ f11b | Card | How do you pick the sketch plane? | Start on the plane (Front, Top or Right) where the profile you will extrude is seen. Choose so the part is oriented as shown in the problem (e.g. the Front plane gives the front view). |  |
| ⚠ f11c | Card | What does "fully defined" mean? | Every sketch entity is fully fixed by dimensions and relations. Fully defined sketch lines are BLACK (under defined = blue, over defined = red). The status bar also tells you. |  |
| ⚠ f11d | Card | Smart Dimension and relations | Smart Dimension sets sizes (length, diameter, angle). Relations (coincident, horizontal, vertical, tangent, equal…) control geometry without numbers. You can show/hide relation icons with View → Sketch Relations. |  |
| ⚠ f11g | Card | Exit and enter sketch mode | Click the sketch / Exit Sketch confirmation icon at the top right (or the sketch check mark). Enter by selecting a plane or face and choosing Sketch, or by editing the sketch in the tree. |  |
| ⚠ f12d | Card | Assign a material and find mass | Right-click Material in the FeatureManager → Edit Material (e.g. Brass). Then Evaluate → Mass Properties to read the mass (the problem states the units). |  |
| ⚠ f12e | Card | What are mates in an assembly? | Relationships between components that position them (coincident, concentric, parallel, distance…). The first component placed is fixed; add mates to remove degrees of freedom of the others. |  |
| ⚠ f13a | Card | Set up a drawing correctly | Sheet size 8.5 × 11 (A), projection = THIRD angle, standard = ANSI. Make 3 views (front, top, right) plus an isometric in the top-right corner. Scale so the views fit well. |  |
| ⚠ f13b | Card | Edit the sheet format / title block | Right-click the sheet → Edit Sheet Format, then fill in name, title/assignment, DWG. NO., weight, material. Exit back to Edit Sheet. |  |
| ⚠ f13d | Card | What does Pack and Go do? | Collects the part, assembly and drawing (and any referenced files) into one folder or ZIP so nothing is missing. File → Pack and Go → Save to Zip file. Submit the ZIP to Kodiak. |  |
| ⚠ t04a | T/F | Ideate is the second step of design thinking. | FALSE. Steps: Empathize, Define, Ideate (3rd), Prototype, Test. |  |
| ⚠ t08b | T/F | It is OK to power a servo directly from a Pico GPIO pin. | FALSE. Use a separate battery pack and share ground; GPIO pins supply only tiny current. |  |
| ⚠ fib04a | Blank | The five design thinking steps are Empathize, ___, Ideate, ___ and Test. | Blank(s): define ¦ prototype Define = problem statement; Prototype = build to learn. |  |
| ⚠ fib09b | Blank | board = WNE103.KitronikPicoMotor() # At set speed 100 the bot travels 12 in/s. # Drive forward 36 inches: board.motorOn(1, 'f', 100) board.motorOn(2, 'f', 100) time.sleep(___) board.motorOff(1) board.motorOff(2) | Blank(s): 3 time = distance ÷ speed = 36 in ÷ 12 in/s = 3 s. |  |
| ⚠ fib09c | Blank | # Pivot right (turn clockwise) in place board.motorOn(1, 'f', 100) board.motorOn(2, ___, 100) time.sleep(0.3) | Blank(s): r Motor 1 forward + motor 2 reverse = pivot right (reverse = 'r'). |  |
| ⚠ mt04a | Match | Empathize | Learn about the user's needs by observing and interviewing |  |
| ⚠ mt04b | Match | Define | Write the problem (need) statement |  |
| ⚠ mt04c | Match | Ideate | Brainstorm many concepts |  |
| ⚠ mt04d | Match | Prototype | Build a quick model of the best concept |  |
| ⚠ mt04e | Match | Test | Try the prototype with users and learn what to change |  |
| ⚠ seq04a | Order | Put the design thinking steps in order: | 1. Empathize 2. Define 3. Ideate 4. Prototype 5. Test (Empathize → Define → Ideate → Prototype → Test (and iterate).) |  |
| ⚠ seq06a | Order | Put the concept selection steps in order: | 1. Prepare the matrix 2. Rate the concepts 3. Rank the concepts 4. Combine and improve 5. Develop the best concept (The goal is not to select the best concept but to develop it.) |  |
| ⚠ seq12a | Order | Order the steps to model a block with a hole: | 1. Choose the sketch plane 2. Sketch the rectangle and dimension it 3. Extruded Boss/Base 4. Sketch a circle on a face 5. Extruded Cut (Base feature first, then add details.) |  |
| ⚠ seq13a | Order | Order the drawing steps: | 1. Make the part and save it 2. New drawing: set 3rd angle, ANSI, 8.5 × 11 3. Insert front, top, right and isometric views 4. Add dimensions, centerlines, and fill the title block 5. Pack and Go to a ZIP (Part first, then the drawing, then package.) |  |
| ⚠ m04a | Quiz | Which step is completed first in design thinking? A) Define B) Empathize C) Ideate D) Test | Answer: B) Empathize You start by understanding the user. |  |
| ⚠ m04b | Quiz | You have drawn three possible scoop concepts and now compare them with a matrix. Which step are you in? A) Empathize B) Define C) Ideate D) Test | Answer: C) Ideate Step 3: Ideate (generate and select concepts). |  |
| ⚠ m04c | Quiz | What are the final two stages of the process? A) Define and Ideate B) Prototype and Test C) Empathize and Define D) Test and Empathize | Answer: B) Prototype and Test Build a prototype then test it with users, then iterate. |  |
| ⚠ m05a | Quiz | Which is the best need (point-of-view) statement? A) Design a stainless steel scoop with a spring lever. B) A restaurant server needs a way to portion ice cream quickly with one hand because lines back up at busy times. C) Make ice cream better. D) Friendly's needs more customers. | Answer: B) A restaurant server needs a way to portion ice cream quickly with one hand because lines back up at busy times. User + need + insight, with no solution built in. |  |
| ⚠ m05c | Quiz | Which is the best CRITERION for success? A) Easy to use B) Cleans in less than 30 seconds using only water and soap C) Looks nice D) Made of metal | Answer: B) Cleans in less than 30 seconds using only water and soap It is specific and measurable. |  |
| ⚠ m06a | Quiz | In a screening matrix, a "+" means the concept is… A) worse than the reference on that criterion B) the same as the reference C) better than the reference on that criterion D) chosen as the final design | Answer: C) better than the reference on that criterion + better, 0 same, − worse. |  |
| ⚠ m06c | Quiz | Concept B scores: Cost +, Cleaning 0, Portion −, Grip +. What is its net score? A) +1 B) 0 C) +2 D) −1 | Answer: A) +1 Two pluses (+2) and one minus (−1) give a net score of +1; the zero adds nothing. |  |
| ⚠ m07d | Quiz | If 3.3 V is placed across a 220 Ω resistor, what current flows (I = V/R)? A) 15 mA B) 1.5 mA C) 150 mA D) 0.15 mA | Answer: A) 15 mA 3.3 ÷ 220 = 0.015 A = 15 mA. |  |
| ⚠ m09d | Quiz | Which statement makes motor 2 spin in reverse at full speed? A) board.motorOn(2, 'r', 100) B) board.motorOn(100, 2, 'r') C) board.motor2.reverse(100) D) board.motorOn(2, reverse) | Answer: A) board.motorOn(2, 'r', 100) motorOn(motor_number, direction, speed). |  |
| ⚠ m09e | Quiz | The bot should run forward for 4 s then stop. Which is correct? A) motorOn(1,'f',100); motorOn(2,'f',100); sleep(4); motorOff(1); motorOff(2) B) sleep(4); motorOn(1,'f',100) C) motorOff(1); motorOn(2,'f',100) D) motorOn(1,'f',4) | Answer: A) motorOn(1,'f',100); motorOn(2,'f',100); sleep(4); motorOff(1); motorOff(2) Start both motors, wait, then stop both. |  |
| ⚠ m11c | Quiz | Which settings should you check when starting a new part for ANSI work? A) Standard = ANSI and Units = IPS B) Standard = ISO and Units = MMGS C) Standard = DIN only D) No settings | Answer: A) Standard = ANSI and Units = IPS Document Properties: ANSI + IPS unless told otherwise. |  |
| ⚠ m12b | Quiz | How do you find the mass of a part? A) Evaluate → Mass Properties (after assigning a material) B) Measure with a ruler C) Insert → Mass D) File → Properties | Answer: A) Evaluate → Mass Properties (after assigning a material) Assign the material first (e.g. brass). |  |
| ⚠ m13b | Quiz | The title block must show… A) Name, assignment/title and DWG. No. B) Only the date C) The file size D) Nothing | Answer: A) Name, assignment/title and DWG. No. Edit the sheet format to fill in name, title, DWG. No., weight and material. |  |

# Part B — Everything else, by topic

## 01 · Graphics: Views & Projection  (36)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f01a | Card | Orthographic vs. perspective projection | Orthographic: viewer is infinitely far away, so projectors are PARALLEL. Measurements stay accurate; used for engineering drawings. Perspective: projectors radiate from one point (the observer), so they are not parallel. Looks realistic but distorts measurements. |  |
| f01b | Card | What is an isometric view? | A type of orthographic projection where the object is turned 45° about the Z axis and tilted about 35° about the X axis so that three sides (front, top and side) show at once. |  |
| f01c | Card | Which two axes does each standard view show? | Front: X–Z Top (horizontal): X–Y Right (profile): Y–Z The front view is seen by a viewer on the Y axis looking toward +Y. |  |
| f01d | Card | Where are the views placed in 3rd angle projection? | Front view in the middle. TOP view directly ABOVE the front. RIGHT view directly to the RIGHT of the front. (Third angle is the ANSI / US standard.) |  |
| f01e | Card | How does 1st angle projection differ from 3rd angle? | In 1st angle the views are flipped: the TOP view is placed BELOW the front view and the RIGHT view is placed to the LEFT of the front view. 1st angle is used in much of Europe/Asia; 3rd angle is used in the US. |  |
| f01f | Card | What are fold (hinge) lines? | The lines where the projection planes meet when the 3D "glass box" is unfolded into a flat sheet. For example HF is the fold line between the Horizontal (top) view and the Front view. |  |
| f01g | Card | A surface can look like three different things in a view. What are they? | 1) A true-shape or foreshortened SURFACE (area) 2) An EDGE VIEW (a line) when the surface is parallel to the line of sight 3) Not visible (hidden behind other geometry; shown with hidden lines if you need it) |  |
| f01h | Card | Lines on a drawing can represent which three features? | 1) The edge view of a surface 2) The intersection of two surfaces 3) The limit (tangent edge) of a curved surface |  |
| t01a | T/F | In 3rd angle projection the right view is placed to the right of the front view. | TRUE. 3rd angle: top view above the front, right view to the right of the front. |  |
| t01b | T/F | In 1st angle projection the top view is placed above the front view. | FALSE. In 1st angle the top view goes BELOW the front and the right view goes to the LEFT. |  |
| t01c | T/F | Perspective projections are used for engineering drawings because they allow accurate measurements. | FALSE. Orthographic (parallel) projection is used because it keeps measurements accurate. |  |
| t01d | T/F | The front view shows the X and Z axes. | TRUE. Front: X–Z. Top: X–Y. Right: Y–Z. |  |
| t01e | T/F | A surface that is parallel to your line of sight appears as an edge (a line). | TRUE. Edge-on surfaces project to lines. |  |
| fib01a | Blank | In ___ angle projection the top view is placed above the front view and the right view is to the ___ of the front view. | Blank(s): third ¦ right 3rd angle is the ANSI standard. |  |
| mt01a | Match | Front view | Shows the X and Z axes |  |
| mt01b | Match | Top view | Shows the X and Y axes |  |
| mt01c | Match | Right view | Shows the Y and Z axes |  |
| mt01d | Match | 3rd angle | Top view above front; right view to the right (ANSI) |  |
| mt01e | Match | 1st angle | Top view below front; right view to the left |  |
| mt01f | Match | Orthographic | Parallel projectors; accurate measurements |  |
| m01a | Quiz | Which projection has the top view BELOW the front view? A) 1st angle B) 3rd angle C) Isometric D) Perspective | Answer: A) 1st angle 1st angle puts the top view below and the right view to the left. |  |
| m01b | Quiz | In an orthographic drawing the projectors are… A) parallel to each other B) all meeting at the observer C) at 45° to the object D) curved | Answer: A) parallel to each other Orthographic = parallel projectors from an infinitely distant viewer. |  |
| m01c | Quiz | The Right view of an object shows which axes? A) X–Y B) X–Z C) Y–Z D) Z only | Answer: C) Y–Z Front: X–Z; Top: X–Y; Right: Y–Z. |  |
| mg0 | Quiz | Surface A (the L-shaped front face) appears in the FRONT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: A) A surface (area) the L-shaped front face faces the front viewer, so you see its area in that view. |  |
| mg1 | Quiz | Surface A (the L-shaped front face) appears in the TOP view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: B) An edge (a line) Surface A is parallel to the line of sight in the Top view, so it shows as an edge (line). |  |
| mg2 | Quiz | Surface B (the top of the tall section) appears in the TOP view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: A) A surface (area) the top of the tall section faces the top viewer, so you see its area in that view. |  |
| mg3 | Quiz | Surface B (the top of the tall section) appears in the FRONT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: B) An edge (a line) Surface B is parallel to the line of sight in the Front view, so it shows as an edge (line). |  |
| mg4 | Quiz | Surface C (the vertical wall of the step) appears in the RIGHT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: A) A surface (area) the vertical wall of the step faces the right viewer, so you see its area in that view. |  |
| mg5 | Quiz | Surface C (the vertical wall of the step) appears in the FRONT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: B) An edge (a line) Surface C is parallel to the line of sight in the Front view, so it shows as an edge (line). |  |
| mg6 | Quiz | Surface D (the top of the step) appears in the TOP view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: A) A surface (area) the top of the step faces the top viewer, so you see its area in that view. |  |
| mg7 | Quiz | Surface D (the top of the step) appears in the RIGHT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: B) An edge (a line) Surface D is parallel to the line of sight in the Right view, so it shows as an edge (line). |  |
| mg8 | Quiz | Surface E (the right end face) appears in the RIGHT view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: A) A surface (area) the right end face faces the right viewer, so you see its area in that view. |  |
| mg9 | Quiz | Surface E (the right end face) appears in the TOP view as… A) A surface (area) B) An edge (a line) C) A hidden line D) It is not visible [Figure: 3-view/iso step-block figure] | Answer: B) An edge (a line) Surface E is parallel to the line of sight in the Top view, so it shows as an edge (line). |  |
| mg-b1 | Quiz | Which is the correct FRONT view outline for this part? A) L-shaped (step at upper right) B) Plain rectangle C) Rectangle with a diagonal line D) Mirror-image L (step at upper left) [Figure: 3-view/iso step-block figure] | Answer: A) L-shaped (step at upper right) Looking from the front you see the stepped profile with the cut at the upper right. |  |
| mg-b2 | Quiz | In the TOP view the vertical line at the middle of the width is… A) The edge between the tall part and the step (visible line) B) A hidden line C) A center line D) The edge of a hole [Figure: 3-view/iso step-block figure] | Answer: A) The edge between the tall part and the step (visible line) The step wall meets the tall top. Both can be seen from above, so it is a visible line. |  |
| mg-b3 | Quiz | In the RIGHT view the horizontal line across the middle shows… A) The edge where the end face and step wall meet (visible) B) A hidden edge C) The center line D) Section lines [Figure: 3-view/iso step-block figure] | Answer: A) The edge where the end face and step wall meet (visible) Surfaces E and C both face the right viewer; they meet at a visible line. |  |

## 02 · Graphics: Lines & Symbols  (35)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f02a | Card | Visible line vs. hidden line | Visible line: thick solid line for edges you can see in that view. Hidden line: medium dashed line for edges that exist but are behind a surface in that view. |  |
| f02b | Card | Center line | Thin line made of long–short–long dashes. Marks the center of holes, cylinders and symmetric features. |  |
| f02c | Card | Phantom line | Thin line of one long dash followed by two short dashes. Shows an alternate position or travel path of a moving part. |  |
| f02d | Card | Section lines and cutting plane line | Section lines: thin diagonal hatching showing material that was cut, giving a look at the interior. Cutting plane line: thick line with arrows at the ends showing where the part was sliced and which way you look. |  |
| f02e | Card | Short break and long break lines | Short break line: thick wavy/freehand line that "breaks out" a portion of a view for clarity. Long break line: thin line with zig-zags used to shorten a long part that has the same shape throughout. |  |
| f02f | Card | Line precedence: when lines coincide, which one is drawn? | Visible > Hidden > Center. A visible line hides a hidden line behind it, and a hidden line takes priority over a center line. Center line tips are still shown sticking out past the edge with a small gap. |  |
| t02a | T/F | A hidden line is a thin, solid line. | FALSE. A hidden line is a medium-weight DASHED line. |  |
| t02b | T/F | When a visible line and a hidden line coincide, the visible line is drawn. | TRUE. Precedence: visible > hidden > center. |  |
| t02c | T/F | A phantom line is used to show an alternate position of a moving part. | TRUE. Long-short-short dash pattern showing travel or alternate positions. |  |
| t02d | T/F | Section lines are the arrows that show where a part is cut. | FALSE. Those are the cutting plane line. Section lines are the thin diagonal hatching on the cut surface. |  |
| fib02a | Blank | When a visible line, a hidden line and a center line coincide, the ___ line is drawn. | Blank(s): visible Visible > hidden > center. |  |
| mt02a | Match | Visible line | Thick solid line for edges you can see |  |
| mt02b | Match | Hidden line | Medium dashed line for edges behind a surface |  |
| mt02c | Match | Center line | Long-short-long dash; marks centers of holes |  |
| mt02d | Match | Phantom line | Long-short-short dash; alternate position of part |  |
| mt02e | Match | Section lines | Thin diagonal hatch on cut surfaces |  |
| mt02f | Match | Cutting plane line | Thick dashed line with arrows showing where a part is sliced |  |
| mt02g | Match | Long break line | Thin line with zig-zag; shortens a uniform part |  |
| mt02h | Match | Short break line | Thick wavy line that breaks out part of a view |  |
| m02a | Quiz | Which line type is used for a hole's axis? A) Hidden B) Center C) Phantom D) Cutting plane | Answer: B) Center Long-short-long. |  |
| m02b | Quiz | In the front view the dashed lines (1) of this part with a vertical hole represent… A) Visible edges B) The hole's edges, which are hidden behind the surface C) Center lines D) Section lines [Figure: hole/hidden-line figure] | Answer: B) The hole's edges, which are hidden behind the surface Hidden lines are dashed and show edges you cannot see from that side. The hole is inside the block. |  |
| m02c | Quiz | In the TOP view the hole (2) appears as… A) A circle (true shape) B) A dashed rectangle C) Not shown D) A phantom line [Figure: hole/hidden-line figure] | Answer: A) A circle (true shape) The hole axis is vertical, so looking from the top you see the hole's circle. |  |
| m02d | Quiz | Line (1) in the front view and line (3) in the right view are both… A) Hidden lines B) Visible lines C) Dimension lines D) Phantom lines [Figure: hole/hidden-line figure] | Answer: A) Hidden lines Both pairs of dashed lines are hidden lines for the same hole. |  |
| ml00 | Quiz | What type of line is line A? A) Visible line B) Long break line C) Short break line D) Hidden line [Figure: line-type chart] | Answer: A) Visible line Line A is a visible line. |  |
| ml01 | Quiz | What type of line is line B? A) Long break line B) Short break line C) Section lines D) Hidden line [Figure: line-type chart] | Answer: D) Hidden line Line B is a hidden line. |  |
| ml02 | Quiz | What type of line is line C? A) Short break line B) Hidden line C) Center line D) Long break line [Figure: line-type chart] | Answer: C) Center line Line C is a center line. |  |
| ml03 | Quiz | What type of line is line D? A) Hidden line B) Phantom line C) Long break line D) Short break line [Figure: line-type chart] | Answer: B) Phantom line Line D is a phantom line. |  |
| ml04 | Quiz | What type of line is line E? A) Cutting plane line B) Long break line C) Short break line D) Hidden line [Figure: line-type chart] | Answer: A) Cutting plane line Line E is a cutting plane line. |  |
| ml05 | Quiz | What type of line is line F? A) Long break line B) Short break line C) Hidden line D) Section lines [Figure: line-type chart] | Answer: D) Section lines Line F is a section lines. |  |
| ml10 | Quiz | What type of line is line A? A) Short break line B) Hidden line C) Dimension line D) Long break line [Figure: line-type chart 2] | Answer: C) Dimension line Line A is a dimension line. |  |
| ml11 | Quiz | What type of line is line B? A) Hidden line B) Extension line C) Long break line D) Short break line [Figure: line-type chart 2] | Answer: B) Extension line Line B is a extension line. |  |
| ml12 | Quiz | What type of line is line C? A) Long break line B) Short break line C) Hidden line D) Section lines [Figure: line-type chart 2] | Answer: A) Long break line Line C is a long break line. |  |
| ml13 | Quiz | What type of line is line D? A) Long break line B) Hidden line C) Section lines D) Short break line [Figure: line-type chart 2] | Answer: D) Short break line Line D is a short break line. |  |
| ml14 | Quiz | What type of line is line E? A) Short break line B) Hidden line C) Visible line D) Long break line [Figure: line-type chart 2] | Answer: C) Visible line Line E is a visible line. |  |
| ml15 | Quiz | What type of line is line F? A) Section lines B) Hidden line C) Long break line D) Short break line [Figure: line-type chart 2] | Answer: B) Hidden line Line F is a hidden line. |  |

## 03 · Graphics: Dimensioning  (24)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f03a | Card | Parts of a dimension | Dimension number, dimension line (with arrowheads), extension lines (show what is being measured and move the dimension away from the part) and leader lines (link a number to a feature, usually circles). |  |
| f03b | Card | ANSI dimension text rule | ANSI (unidirectional) style: ALL dimension numbers are horizontal so they can be read without turning the page. ANSI goes with 3rd angle projection. |  |
| f03c | Card | What does tolerance mean? | The maximum amount a dimension is allowed to vary = max size − min size. Example: Ø0.750 ± .005 means the hole can be .745 to .755, a total tolerance of .010. |  |
| f03d | Card | Ø and R symbols | Ø = diameter (used on holes and cylinders with a leader line). R = radius (used on rounded corners and arcs). |  |
| f03e | Card | Dimensioning best practices | • Put dimensions between views, grouped on 1–2 views • Keep intermediate dimensions INSIDE the overall dimension • Dimension each feature once (no over-dimensioning), and don't leave sizes or locations out (under-dimensioning) • Use extension lines; keep dimension lines off the part |  |
| f03f | Card | Over-dimensioned vs. under-dimensioned | Over: the same size is given twice (or in a closed chain that includes the overall), so the dimensions conflict. Under: a size or location is missing, so the part cannot be made from the drawing. |  |
| f03g | Card | What does an underlined dimension number mean? | The dimension is NOT drawn to scale (the number was changed without redrawing the geometry). |  |
| t03a | T/F | In ANSI style, dimension numbers should all be horizontal. | TRUE. Unidirectional ANSI text makes drawings easy to read. |  |
| t03b | T/F | Good practice is to put dimensions on the outside of all three views, spread over every view. | FALSE. Dimensions go between views and are grouped on 1–2 views. |  |
| t03c | T/F | An intermediate dimension should be placed inside the overall dimension. | TRUE. Overall dimension outermost, shorter ones inside, to keep it tidy. |  |
| t03d | T/F | A drawing that gives 2.00 + 2.00 and also the overall 4.00 for the same length is correctly dimensioned. | FALSE. That is over-dimensioning (redundant). Leave out one of them. |  |
| t03e | T/F | The symbol Ø in front of a number indicates a radius. | FALSE. Ø is diameter. R is radius. |  |
| t03f | T/F | A dimension of Ø0.750 ± .005 has a total tolerance of 0.010. | TRUE. It may range from .745 to .755 so max − min = .010. |  |
| fib03a | Blank | The ___ line links a dimension number to a circle, and ___ lines extend from the feature to the dimension. | Blank(s): leader ¦ extension Leader lines point to circular features; extension lines locate the edges being measured. |  |
| mt03a | Match | Extension line | Thin line that extends from a feature to the dimension line |  |
| mt03b | Match | Leader line | Line with arrow that links a note to a feature (often a circle) |  |
| mt03c | Match | Tolerance | Max size − min size of a dimension |  |
| mt03d | Match | Ø | Diameter symbol |  |
| mt03e | Match | R | Radius symbol |  |
| m03a | Quiz | Which panel is dimensioned BEST? (hole must be sized and located once, with horizontal numbers) A) A B) B C) C D) D [Figure: dimensioning panels A–D] | Answer: A) A A has overall size, hole size (Ø) and hole location. B is over-dimensioned (2.00 + 2.00 + 4.00). C is missing the hole diameter. D has rotated numbers and dimension lines through the part with no extension lines. |  |
| m03b | Quiz | What is the main error in panel B? A) Over-dimensioned (a redundant 2.00 + 2.00 and 4.00) B) The hole size is missing C) Text is not horizontal D) Wrong projection [Figure: dimensioning panels A–D] | Answer: A) Over-dimensioned (a redundant 2.00 + 2.00 and 4.00) Three dimensions on one length where two would define it. |  |
| m03c | Quiz | What is the main error in panel C? A) The hole diameter is missing B) Over-dimensioned C) No extension lines D) The part is not drawn [Figure: dimensioning panels A–D] | Answer: A) The hole diameter is missing You could not make the hole: its size is not given (under-dimensioned). |  |
| m03d | Quiz | What is the main error in panel D? A) Dimension lines drawn through the part, no extension lines and rotated text B) Hole location missing C) Over-dimensioned hole D) The Ø symbol is missing [Figure: dimensioning panels A–D] | Answer: A) Dimension lines drawn through the part, no extension lines and rotated text Dimensions belong outside the part with extension lines, with horizontal numbers. |  |
| m03e | Quiz | What does ± .005 on a dimension tell you? A) The size may vary by 0.005 either side of nominal B) The part must be exactly nominal C) The dimension is not to scale D) The number is a radius | Answer: A) The size may vary by 0.005 either side of nominal Total tolerance is 0.010. |  |

## 04 · Design Thinking: The 5 Steps  (4)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f04g | Card | Looks-like vs. works-like prototypes | Looks-like (appearance model): shows visuals and feel. Works-like (functional): demonstrates feasibility. Sometimes both. Your rescue bot is a works-like prototype. |  |
| t04b | T/F | After testing a prototype you may need to go back to an earlier step. | TRUE. Design thinking is iterative. |  |
| t04c | T/F | Empathize means starting by building the best solution you can think of. | FALSE. Empathize is about understanding the user first. |  |
| t04d | T/F | A works-like prototype demonstrates function and feasibility. | TRUE. Looks-like shows appearance; works-like shows function. |  |

## 05 · Need, Constraints & Criteria  (10)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f05d | Card | Constraint vs. criterion: quick test | If failing it eliminates the design, it is a CONSTRAINT. If it lets you rank one design as better than another, it is a CRITERION. |  |
| t05a | T/F | A need statement should describe the solution you plan to build. | FALSE. It states the user's need and insight, not the solution. |  |
| t05b | T/F | "The scoop shall cost less than $25 per unit" is a constraint. | TRUE. It is a pass/fail limit given in the problem. |  |
| t05c | T/F | Criteria for success are used to compare how well different concepts perform. | TRUE. Constraints eliminate; criteria rank. |  |
| t05d | T/F | "The scoop should be nice" is a good criterion for success. | FALSE. Too vague. Criteria should be specific and measurable. |  |
| fib05a | Blank | A ___ is a hard limit a design must meet, while a ___ for success measures how well it performs. | Blank(s): constraint ¦ criterion Constraints eliminate designs; criteria compare them. |  |
| mt05a | Match | Need statement | User + need + insight; not a solution |  |
| mt05b | Match | Constraint | Pass/fail limit such as cost < $25 |  |
| mt05c | Match | Criterion for success | Measurable way to judge how good a design is |  |
| m05b | Quiz | Which of these is a CONSTRAINT? A) The scoop shall cost less than $25 per unit. B) The scoop should feel comfortable. C) The scoop would be fun to use. D) The scoop looks cool. | Answer: A) The scoop shall cost less than $25 per unit. Cost limit is pass/fail and stated in the problem. |  |

## 06 · Concept Screening Matrix  (8)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f06d | Card | What is the real goal of concept selection? | Not just to select the best concept, but to DEVELOP the best concept. Combine the good features of different concepts and refine them. |  |
| t06a | T/F | In a screening matrix, concepts are rated relative to a reference concept using +, 0 and −. | TRUE. A datum concept is chosen and every other concept is compared to it. |  |
| t06b | T/F | The only goal of concept selection is to pick one winner and throw the others away. | FALSE. Combine and improve concepts to DEVELOP the best one. |  |
| t06c | T/F | A scoring matrix uses weights for the criteria. | TRUE. Weighted score = Σ(weight × rating). |  |
| fib06a | Blank | In a screening matrix each concept is rated against the ___ concept using +, 0 or −, then the net scores are used to ___ the concepts. | Blank(s): reference ¦ rank Reference (datum) concept; rank, combine and improve. |  |
| mt06a | Match | Screening matrix | + / 0 / − ratings against a reference concept |  |
| mt06b | Match | Scoring matrix | Weighted 1–5 ratings |  |
| m06b | Quiz | After ranking concepts with the matrix, what should you do? A) Throw away everything except the winner B) Combine and improve the best features, then develop the best concept C) Start prototyping all of them D) Skip to testing | Answer: B) Combine and improve the best features, then develop the best concept Goal: develop the best concept. |  |

## 07 · Breadboard & Circuit Basics  (21)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f07b | Card | How do you avoid a short circuit? | Never connect + directly to − (or a supply directly to ground) without a load such as a resistor, LED or motor in between. Don't let bare leads of two different strips touch. Check wiring before powering. |  |
| f07c | Card | What does LED stand for? Polarity? | Light Emitting Diode. Current only flows one way: anode (+, longer leg) to cathode (−, flat side / shorter leg). Installed backwards, it stays dark. |  |
| f07d | Card | Why does an LED need a resistor? | To limit current. An LED has almost no resistance once it conducts, so without a resistor the current can destroy the LED (and the Pico pin). |  |
| f07e | Card | Ohm's Law and units | V = I × R V = voltage (volts, V) I = current (amperes, A) R = resistance (ohms, Ω) |  |
| f07f | Card | Digital vs. analog signals | Digital: discrete, only two levels, HIGH or LOW (on/off). Blinking an LED uses a digital signal. Analog: continuous, any value in a range (e.g. a temperature sensor voltage). |  |
| t07a | T/F | The holes in one column of five (a–e) on the same row of a breadboard are connected. | TRUE. Each 5-hole strip is connected; the center gap separates the halves. |  |
| t07b | T/F | Holes a–e and f–j of the same row are connected across the center gap. | FALSE. The center gap isolates the two halves. |  |
| t07c | T/F | An LED works equally well connected in either direction. | FALSE. An LED conducts in one direction only (anode +, cathode −). |  |
| t07d | T/F | LED stands for Light Emitting Diode. | TRUE. A diode that emits light. |  |
| t07e | T/F | Connecting a supply + terminal directly to − with a wire can create a short circuit. | TRUE. No load to limit current: wires, battery and parts can overheat. |  |
| fib07a | Blank | Ohm's law: V = ___ × R, with resistance measured in ___. | Blank(s): I ¦ ohms V = IR; unit symbol Ω. |  |
| mt07a | Match | Ω | Unit symbol for resistance |  |
| mt07b | Match | Ampere | SI unit for electrical current |  |
| mt07c | Match | Voltage | Electrical potential |  |
| mt07d | Match | Analog | Continuous, infinitely varying signal |  |
| mt07e | Match | Digital | Discrete (HIGH or LOW) signal |  |
| mt07f | Match | R | Variable for resistance in Ohm's law |  |
| mt07g | Match | I | Variable for current in Ohm's law |  |
| m07a | Quiz | Which hole is electrically connected to the gold hole (row 3, column b)? A) Row 3, column e B) Row 3, column f C) Row 4, column b D) Row 3, column j [Figure: breadboard strip figure] | Answer: A) Row 3, column e Holes a–e of the same row are one strip. The center gap separates f–j. |  |
| m07b | Quiz | What is the unit symbol for resistance? A) A B) V C) Ω D) W | Answer: C) Ω Ohm = Ω. |  |
| m07c | Quiz | A signal that is continuous and can take any value in a range is… A) digital B) analog C) binary D) PWM | Answer: B) analog Digital = HIGH or LOW only. |  |

## 08 · Wiring: LED, Servo & Motors  (22)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f08a | Card | Wire an LED off a digital pin | GPIO pin → resistor (about 220 Ω) → LED anode (long leg); LED cathode (short leg / flat side) → GND. In code: led = Pin(15, Pin.OUT); led.on() |  |
| f08c | Card | Why must the servo supply and the Pico share a ground? | The signal voltage is measured relative to ground. Without a common GND the servo has no reference and ignores the Pico's signal. |  |
| f08d | Card | Standard vs. continuous servo | Standard: signal sets the shaft POSITION (movement range of about 90° or 180°). Continuous: signal sets direction and SPEED of rotation, not position. |  |
| f08g | Card | Troubleshooting a physical setup: checklist | 1) Power on? (battery voltage, shield switch) 2) Ground connected / common? 3) Right pin number in the code vs. the wire? 4) LED backwards? Resistor present? 5) Loose or wrong-strip wires? Breadboard rows? |  |
| t08a | T/F | A servo's signal wire is usually orange or yellow. | TRUE. Red = V+, brown or black = GND. |  |
| t08c | T/F | A standard servo controls shaft position. | TRUE. A continuous servo controls speed and direction. |  |
| t08d | T/F | The motor shield must be switched ON for the motors to run. | TRUE. Also remember to turn it off afterwards to save the batteries. |  |
| t08e | T/F | The Pico GPIO pin outputs a logic HIGH of about 3.3 V. | TRUE. Pico logic level is 3.3 V. |  |
| fib08a | Blank | A servo has three wires: signal, ___ (red) and ___ (brown or black). | Blank(s): V+ ¦ GND Power and ground; signal is orange or yellow. |  |
| mt08a | Match | Servo signal wire | Orange or yellow; connects to a GPIO pin |  |
| mt08b | Match | Servo V+ wire | Red; goes to the battery pack + |  |
| mt08c | Match | Servo GND wire | Brown or black; common ground with the Pico |  |
| mt08d | Match | Standard servo | Signal sets shaft position |  |
| mt08e | Match | Continuous servo | Signal sets speed and direction |  |
| mt08f | Match | PWM | Controls motor speed by switching power on and off quickly |  |
| mt08g | Match | H-bridge | Circuit that reverses motor direction |  |
| m08a | Quiz | GP15 is set HIGH. Which circuits light the LED? (All have a resistor.) A) A only B) A and D C) B and C D) All four [Figure: LED circuits A–D] | Answer: B) A and D A and D are complete paths with the LED facing the right way (the order of resistor and LED does not matter). B has the LED backwards; C has no connection to GND. |  |
| m08b | Quiz | What colour is a servo's signal wire (usually)? A) Black B) Red C) Orange or yellow D) Brown | Answer: C) Orange or yellow Red = V+, brown/black = GND. |  |
| m08c | Quiz | The servo does not move. Looking at the wiring, what is wrong? A) The servo signal is on the wrong colour wire B) There is no common ground between the Pico and the battery pack C) The battery pack is too small D) GP15 cannot make a signal [Figure: servo wiring (no common ground)] | Answer: B) There is no common ground between the Pico and the battery pack Battery − must connect to a Pico GND so the signal has a reference. |  |
| m08d | Quiz | What is the main advantage of a breadboard? A) Built-in resistors B) Permanent connections C) Easy prototyping without soldering D) Higher current capacity | Answer: C) Easy prototyping without soldering Parts can be changed quickly with no solder. |  |
| m08e | Quiz | Which controls the direction of a DC motor on the shield? A) A voltage regulator B) An H-bridge C) A resistor D) A servo | Answer: B) An H-bridge The H-bridge reverses current; PWM sets the speed. |  |
| m08f | Quiz | You see this wiring and the servo works. Which statement is TRUE? A) Signal from GP15, power from a separate pack, grounds joined B) The servo is powered by GP15 C) The battery + goes to the Pico D) No ground is needed [Figure: servo wiring (correct)] | Answer: A) Signal from GP15, power from a separate pack, grounds joined This is the recommended servo wiring. |  |

## 09 · Python on the Pico  (22)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f09a | Card | Python syntax rules to remember | • # starts a comment • Indentation defines blocks (after a colon :) • Case-sensitive (Pin ≠ pin, True ≠ true) • No type declarations and no semicolons • Code runs top to bottom; no main() needed |  |
| f09b | Card | Set up a pin for writing (output) | from machine import Pin led = Pin(15, Pin.OUT) led.on() # or led.value(1) led.off() # or led.value(0) |  |
| f09d | Card | Delays and loops | import time time.sleep(0.5) # wait 0.5 s while True: # repeat forever (indented block) for i in range(10): # repeat 10 times |  |
| f09g | Card | Debugging a program: where to look | 1) Read the error message and line number 2) Indentation and colons 3) Capitalization and spelling of names 4) Missing import or wrong pin number 5) Missing sleep (action too fast to see) |  |
| t09a | T/F | In Python you must declare a variable's type before using it. | FALSE. Python is dynamically typed. Just assign. |  |
| t09b | T/F | Indentation matters in Python. | TRUE. Indentation defines which lines belong to a loop, if, or function. |  |
| t09c | T/F | Every Python statement must end with a semicolon. | FALSE. Python does not use semicolons to end statements. |  |
| t09d | T/F | Python code cannot run without a main() function. | FALSE. Code runs top to bottom without main(). |  |
| t09e | T/F | You have to specify if a GPIO pin is an input or an output. | TRUE. e.g. Pin(15, Pin.OUT). |  |
| t09f | T/F | `pin(15, Pin.OUT)` and `Pin(15, Pin.OUT)` are equivalent in Python. | FALSE. Python is case-sensitive: the class is Pin. |  |
| t09g | T/F | The Pico can read an analog voltage on GP26. | TRUE. GP26–GP28 are ADC pins. |  |
| fib09a | Blank | from machine import Pin import time led = Pin(15, Pin.___) while True: led.___() time.sleep(0.5) | Blank(s): OUT ¦ toggle Pin.OUT makes the pin an output; toggle() flips the LED each pass. (led.on() / led.off() also work.) |  |
| fib09d | Blank | ___ Set up the LED pin led = Pin(15, Pin.OUT) for i in range(5):___ led.toggle() | Blank(s): # ¦ : # begins a comment; a colon ends the for-line, and the body is indented. |  |
| mt09a | Match | Module | Group of functions, classes and statements that simplifies code |  |
| mt09b | Match | # | Starts a comment in Python |  |
| mt09c | Match | Pin.OUT | Sets a GPIO pin up for writing |  |
| mt09d | Match | time.sleep(2) | Pauses the program for two seconds |  |
| mt09e | Match | main.py | The file the Pico runs automatically at power-up |  |
| seq09a | Order | Order the steps to blink an LED in Python: | 1. import the Pin class and time 2. Create the pin as an output 3. Start a while True: loop 4. Turn the LED on / toggle 5. Sleep for a moment (Imports → set up pins → loop → act → delay.) |  |
| m09a | Quiz | Which line sets GP15 up as an output? A) led = pin(15, OUT) B) led = Pin(15, Pin.OUT) C) led = Pin(15, Pin.OUT); D) Pin led(15, OUT) | Answer: B) led = Pin(15, Pin.OUT) Capital P, Pin.OUT, no semicolon. |  |
| m09b | Quiz | A program has `while True:` followed by `led.on()` and `time.sleep(1)`, both starting at the far-left margin. What is wrong? A) Nothing B) The loop body is not indented C) sleep needs a semicolon D) while is invalid | Answer: B) The loop body is not indented The lines under while must be indented. |  |
| m09c | Quiz | Which Python line is a comment? A) // turn on LED B) # turn on LED C) -- turn on LED D) /* turn on LED */ | Answer: B) # turn on LED # starts a comment. |  |

## 10 · Motor Data, Excel & Memo  (14)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f10a | Card | Translational velocity from wheel data | v = (revolutions × wheel circumference) / time interval Circumference = π × diameter. Units: inches per second. |  |
| f10b | Card | Using the motor characterization to plan a path | Look up the speed (in/s) for your set speed value, then time = distance ÷ speed. Use that time in time.sleep(). |  |
| f10d | Card | Which variable goes on which axis? | The thing you control or change (independent variable, e.g. speed value) goes on the X axis. The thing you measure (dependent, e.g. velocity) goes on the Y axis. |  |
| t10a | T/F | In a scatter plot of the motor data, the speed setting you choose goes on the X axis. | TRUE. Independent variable on X; measured velocity on Y. |  |
| t10b | T/F | A memo needs TO, FROM, DATE and SUBJECT lines. | TRUE. Standard memo header. |  |
| fib10a | Blank | Translational velocity = (___ × wheel circumference) ÷ time. | Blank(s): revolutions Circumference = π × diameter. |  |
| mt10a | Match | v = (rev × C) / t | Translational velocity from wheel data |  |
| mt10b | Match | C = π d | Wheel circumference |  |
| mt10c | Match | t = d / v | Time to travel a distance at a known speed |  |
| mt10d | Match | Memo header | TO, FROM, DATE, SUBJECT |  |
| mt10e | Match | X axis of scatter plot | The variable you set (speed value) |  |
| m10a | Quiz | Speed value 80 gives 8 in/s. How long to travel 24 in? A) 3 s B) 2 s C) 0.33 s D) 192 s | Answer: A) 3 s time = distance ÷ speed = 24 ÷ 8 = 3 s. |  |
| m10b | Quiz | A wheel of diameter 2.5 in turns 10 times in 5 s. Translational velocity is about… A) 15.7 in/s B) 7.85 in/s C) 31.4 in/s D) 5 in/s | Answer: A) 15.7 in/s C = π × 2.5 = 7.85 in. 10 rev × 7.85 in ÷ 5 s = 15.7 in/s. |  |
| m10c | Quiz | Which chart type is used for the motor characterization data? A) Pie chart B) Scatter plot C) Bar chart of names D) Histogram | Answer: B) Scatter plot Numeric X vs. numeric Y. |  |

## 11 · SolidWorks: Sketching  (9)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f11e | Card | Construction lines and Mirror Entities | Construction geometry guides the sketch and is not used to make the feature (a centerline for symmetry). Mirror Entities mirrors sketched geometry about a centerline/construction line. |  |
| f11f | Card | Trim Entities | Removes the portion of a sketch line or arc between intersections; used to clean up overlapping geometry. |  |
| t11a | T/F | A fully defined sketch is shown in black. | TRUE. Blue = under defined, red = over defined. |  |
| t11b | T/F | Construction lines become part of the extruded profile. | FALSE. Construction geometry is a guide only. |  |
| fib11a | Blank | Fully defined sketch geometry is shown in ___; under defined geometry is shown in blue. | Blank(s): black Red indicates over defined. |  |
| mt11a | Match | Smart Dimension | Sets sizes of sketch entities |  |
| mt11b | Match | Trim Entities | Removes sketch segments between intersections |  |
| m11a | Quiz | A sketch entity is blue. It is… A) fully defined B) under defined C) over defined D) a construction line only | Answer: B) under defined Black = fully defined; red = over defined. |  |
| m11b | Quiz | Which tool mirrors sketch geometry about a centerline? A) Trim Entities B) Mirror Entities C) Sketch Fillet D) Offset | Answer: B) Mirror Entities Mirror in sketch mode, using a construction line. |  |

## 12 · SolidWorks: Features & Assemblies  (13)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f12a | Card | Extruded Boss/Base vs. Extruded Cut | Boss/Base ADDS material by pushing a sketch profile out to a depth. Cut REMOVES material (holes, pockets) from the existing solid. |  |
| f12b | Card | Fillet vs. chamfer | Fillet: rounds an edge (radius). Chamfer: bevels an edge (flat angled cut, distance/angle). |  |
| f12c | Card | Linear pattern vs. circular pattern | Linear: repeats a feature along one or two directions with a spacing and count. Circular: repeats a feature around an axis with a count and total angle (e.g. 360°). |  |
| t12a | T/F | A fillet bevels an edge with a flat cut. | FALSE. That is a chamfer. A fillet rounds an edge. |  |
| t12b | T/F | A circular pattern repeats a feature around an axis. | TRUE. Linear patterns repeat along a direction. |  |
| fib12a | Blank | An ___ Cut removes material, while a ___ rounds an edge. | Blank(s): extruded ¦ fillet Chamfer bevels an edge. |  |
| mt12a | Match | Extruded Boss/Base | Adds material from a sketch |  |
| mt12b | Match | Extruded Cut | Removes material from a solid |  |
| mt12c | Match | Fillet | Rounds an edge |  |
| mt12d | Match | Chamfer | Bevels an edge |  |
| mt12e | Match | Mate | Relationship that positions components in an assembly |  |
| m12a | Quiz | Which feature removes material? A) Extruded Boss/Base B) Extruded Cut C) Linear Pattern D) Mirror only | Answer: B) Extruded Cut Cut removes. |  |
| m12c | Quiz | Which pattern would you use to place 6 holes equally around a circle? A) Linear pattern B) Circular pattern C) Chamfer D) Mirror | Answer: B) Circular pattern Circular pattern: count 6 over 360°. |  |

## 13 · SolidWorks: Drawings & Pack and Go  (9)

| ID | Type | Question / prompt | Answer & explanation | Check |
| --- | --- | --- | --- | --- |
| f13c | Card | What must a good drawing show? | Hidden lines and center lines, correctly placed and complete dimensions (not over- or under-dimensioned), neat layout, and a filled title block. |  |
| t13a | T/F | The drawing should be in 1st angle projection for ANSI. | FALSE. ANSI uses THIRD angle projection. |  |
| t13b | T/F | Pack and Go gathers the part and drawing files into one ZIP. | TRUE. Submit that ZIP to Kodiak. |  |
| fib13a | Blank | Use ___ and Go to save the part, drawing and assembly into one ZIP file. | Blank(s): pack File → Pack and Go. |  |
| mt13a | Match | .SLDPRT | SolidWorks part file |  |
| mt13b | Match | .SLDDRW | SolidWorks drawing file |  |
| mt13c | Match | .SLDASM | SolidWorks assembly file |  |
| m13a | Quiz | Which projection must the midterm drawing use? A) 1st angle B) 3rd angle C) Isometric only D) Perspective | Answer: B) 3rd angle Third angle, ANSI, 8.5 × 11. |  |
| m13c | Quiz | Pack and Go is used to… A) create a zipped folder with the part and drawing B) print the drawing C) change units D) apply a material | Answer: A) create a zipped folder with the part and drawing Submit the ZIP to Kodiak. |  |